What is a Subnet?

A subnet is a smaller network within a larger IP network. It helps divide a large network into smaller, more manageable pieces. This improves performance, enhances security, and simplifies management.

🔹 Why Use Subnets?

Efficient IP address usage

Improved network performance

Enhanced security

Simplified troubleshooting

🔹 Key Concepts

Term	Description

IP Address	A unique address for each device on a network (e.g., 192.168.1.10)

Subnet Mask	Determines which part of the IP address is the network and which is the host (e.g., 255.255.255.0)

Network Address	The first address in a subnet (e.g., 192.168.1.0)

Broadcast Address	The last address in a subnet (used to communicate with all devices in the subnet)

Host Address	Addresses assigned to devices in the subnet (between network and broadcast addresses)

🔹 Example

Say you have an IP address: 192.168.1.100

With a subnet mask: 255.255.255.0

Network Address: 192.168.1.0

Broadcast Address: 192.168.1.255

Host Range: 192.168.1.1 to 192.168.1.254

Total Hosts: 254 (2^8 - 2 = 256 - 2)

🔹 CIDR Notation (Classless Inter-Domain Routing)

Instead of writing the full subnet mask, you can use CIDR:

192.168.1.0/24 → means 24 bits are for the network (255.255.255.0)

192.168.1.0/26 → smaller subnet (255.255.255.192), only 64 addresses

🔹 Subnetting Formula

To calculate hosts and subnets:

Number of Hosts per Subnet: 2^(32 - subnet bits) - 2

Number of Subnets: 2^(borrowed bits)

🔹 Common Subnet Masks

CIDR	Subnet Mask	Hosts

/30	255.255.255.252	2	Point-to-point links

/29	255.255.255.248	6	Small subnets

/24	255.255.255.0	254	Standard class C

/16	255.255.0.0	65,534	Large networks

🔹 Real-World Uses

Isolating departments (HR, IT, etc.)

Improving security using firewalls between subnets

Managing IP address space efficiently

Dividing networks in large organizations or data centers

Would you like help calculating a specific subnet or visualizing subnet divisions?